Contents

Cautions for Use	1

Product parts and installation	3

1. Parts and Accessories	3

2. Specifications	4

3. Description of MAGIC-1S	5

4. Installation	7

(1) MagicArt Program Installation	7

(2) MAGIC-1S installation	8

5. Engraver movement	9

(1) The order of self-test when the machine is turned on	9

(2) The sequence of instruction for engraving set up	9

6. How to use the clamp	11

(1) Holding method for outer diameter engraving	11

(2) Holding method for ring inner diameter engraving	11

(3) Chuck finger changing method	12

(4) Materials holding methods for engraving bracelets	12

7. Changing tools	13

8. Adjusting tension	13

Program Usage Description	14

1) Toolbar	16

2) Text object property	37

3) Rotation tool property	43

4) How to engrave for each toolpath	44

5) Create Toolpath	49

6) Toolpath object property	54

7) NC data output	56

Engraving Process of Ring inner/outer Diameter	59

Engraving Curved Materials	66

Layout of Text in a Circle	67

Layout of Text on a Curve	69

Font Preview	70

Text Change	71

Template Creation	72

Saving as Design Sample	77

Auto Change of Serial Number	80

Loading and Editing Image	83

1) Loading an image	83

2) Converting and editing image	85

Import a File in SVG Type	88

Origin point calibration	90

Cautions for Use

Please be sure to read and follow the instructions below for safety use.

- The instructions below are intended to prevent personal injury and property damages.

- Please read carefully and use the machine properly.

|  | If you do not follow these instructions, injury or property damage may occur. |
| --- | --- |
|  | If you do not follow these instructions, users may be seriously injured. |

| - Cautions related to power |  |  |  |
| --- | --- | --- | --- |
|  | Do not use damaged cord or loosened outlet. - It may cause electric shock or fire. |  | Do not put multiple power plugs into one inlet simultaneously. It may cause fire due to overheated inlet. |  |
|  | Do not pull a plug by the cord. Do not touch a power plug with wet hands. |  | When the product is not in use for a long term, pull out the power plug. |  |
|  | If smoke comes out from the engraver during use, turn off the engraver immediately and call the sales agency or our service center. |  |  |  |

| - Cautions related to use |  |  |  |
| --- | --- | --- | --- |
|  | Do not use this machine in a place with oil, smoke, humidity, dust or water. - It may cause electric shock or fire. |  | Do not put a candle or cigarette butt on the engraver. Avoid any heating apparatus like heater when using this machine. - It may cause failure or fire due to overheating. |
|  | Be sure to place the engraver body only on a stable level surface. - The engraver weighs about 28kg, and it vibrates left and right during operation. Therefore, if you use this machine on an uneven surface, it may cause damage to the surface and products, and injury to users. |  |  |

| - Cautions related to use |  |  |  |
| --- | --- | --- | --- |
|  | Do not allow any body parts or other objects onto the engraver. - The cutter of engraver may cause serious injury during its operation. |  | Do not put water or small metal materials on the engraver. - Due to negligence or vibration of the engraver, the materials may fall and cause injury, fire or failure by overheating or short circuiting. - If water gets into the product, it may cause electric shock or fire. |
|  | Be sure not to be caught or jammed between the paths of each axis of the engraver, or scratched by the engraving cutter. - It may cause serious injury, or damage or failure of the engraver. |  | Laser Radiation Avoid eye or skin exposure to direct or scattered radiation. Class IIIA laser product. |

| - Other cautions |  |  |  |
| --- | --- | --- | --- |
|  | Do not disassemble or modify the engraver. - It may cause fire, electric shock or failure. Please contact the sales agency or our service center for Inspection, calibration and repair. |  | Be sure to keep out of the reach of toddlers and children. Their curiosity or carelessness may cause injury. |
|  | When temperate falls below freezing point, the product may not normally perform. In this case, please sustain the room temperature in the place where you use this engraver for normal operation. Please make sure the temperature in the place where you use the engraver does not fall below the freezing point. |  | We will not be held responsible for damage caused to product by misuse or poor care. |

Product parts and installation

1. Parts and Accessories

MAGIC-1S product box includes the engraver and following accessories.

| Accessories | Photos | Qty | Purpose |
| --- | --- | --- | --- |
| 1.5mm L-wrench |  | 1EA | For changing tools |
| Chuck finger skin |  | 6EA | For protect chuck fingers |
| Flat chuck finger |  | 3EA | Jig for holding rings when marking ring’s outer diameter or inner diameter of flat rings |
| Round chuck finger (Wide width) |  | 3EA | Jig for holding rings when marking ring’s inner diameter |
| Round chuck finger (Narrow width) |  | 3EA | Jig for holding rings when marking ring’s inner diameter |
| Chuck handle |  | 2EA | Rotation clamp tightening handle |
| Ruler |  | 1EA |  |
| Finger spanner |  | 1EA | Tool for tightening fingers |
| Power adapter |  | 1EA | DC 24V power supply (24V, 2.5A) |
| USB cable |  | 1EA | For connecting to PCs |
| Program CD |  | 1EA | MagicArt, design and engraving S/W |
| User’s manual |  | 1EA | User’s manual |

2. Specifications

| Power supply | AC 100~240V |
| --- | --- |
| Consumption voltage | 12W |
| Operating range | Rotary: Cylinder outside diameter: 40mm Cylinder inside diameter: 30mm |
| Resolution | 0.005mm (X), 0.00125(Z) |
| Weight | 13kg |
| Transfer speed | 1 – 36mm/sec (X), 1 – 18mm/sec (Z) |
| Temperature | 5 – 40 ºC |
| Humidity | 10 - 90 % |
| Purposes | Ring inner diameter, outer diameter marking |

A. Power switch

E. Rotation clamp

(Chuck)

D. Engraving tool

(Tip)

3. Description of MAGIC-1S

B. Tension adjuster

C. Operation key

F. Power port

G. Communication port

◈ Description of each part

| A | Power switch | Power on/off switch. |
| --- | --- | --- |
| B | Tension adjustor | By rotating knob, tool tension is adjusted. • Clockwise: Gives more tension • Counter clockwise: Gives less tension |
| C | Operation key | Operates engraver’s various functions. |
| D | Engraving tool(Tip) | Tool for engraving. Tip is expendable. |
| E | Rotation clamp(Chuck) | Holds cylinder engraving materials such as rings. |
| F | Power port | Supplies 24V to engraver by connecting to power adapter. |
| G | Communication port | Connects USB cable to computer. |

◈ Description on keys

|  | Stop | Stop engraving during related operation |
| --- | --- | --- |
|  | Pointer | Turns laser pointer ON/OFF. |
|  | Start | If this button is pressed and held for more than one second after engraving, then the data is sent and the area is designated, engraving will start. |
|  | Save | If the button is pressed and held for more than one second after the tool setting is changed, then the changed setting value will be saved. |
|  | Direction key (X, A) | On stand-by state: X-/X+ moves X axis left or right. A+/A- moves A(rotary) axis up or down. On engraving state: Checks and moves the engraving position |
|  | Direction key (Z) | Move Z-axis up and down |

4. Installation

(1) MagicArt Program Installation

Place the CD in your computer`s CD driver, and follow the below instructions to install the S/W. Installed program will be automatically executed and program installation will be progressed in the following order. If Install program is not automatically executed after CD is entered, execute Install program on My Computer’s CD drive.

| 1.Setup wizard | 2. Select installation folder |
| --- | --- |
| 3. Confirm installation | 4. The file is copied onto the hard disk. |
| 5. Click “Close” button to complete program installation. |  |

◈ Program description

Once the program is installed, the icon below will appear on the screen. The description of the icon is as follows.

(2) MAGIC-1S installation

MAGIC-1S is a desktop CNC engraver. This engraver cannot be operated independently. You need your machine to be connected to a PC and run with a provided program, in order to reform engraving work as you want.

USB cable

Power cable to outlet

USB cable to computer

Insert the provided USB cable into the port on the rear side of the engraver and the USB port of your computer respectively.

5. Engraver movement

When the engraver is turned on, the engraver begins the self-test in the following order, and it enters into waiting mode with confirmation sound if there is no problem.

(1) The order of self-test when the machine is turned on

1) If a certain axis is detected by the sensor, move to leave from the detected area.

2) Check Z-axis sensor and return to the starting point

3) Check Y-axis sensor and return to the starting point

4) Check X-axis sensor and return to the starting point

5) Wait for confirmation sound

(2) The sequence of instruction for engraving set up

1) Turn on the engraver.

2) Place to secure the materials to be engraved on the clamp of the engraver.

3) Run the design software (provided with the product).

4) Prepare the contents of engraving.

5) Press the engraving button on the program to transfer intended data to the machine.

6) Press the start button on the machine to begin the engraving process.

6. How to use the clamp

(1) Holding method for outer diameter engraving

Insert the ring until it clicks into the flat chuck finger and turn the chuck handle to hold the ring and not to allow it come out or spin.

(2) Holding method for ring inner diameter engraving

Open the chuck finger of the rotation clamp and insert the ring, and use the chuck handle to hold it not to allow it to come out or slip.

Ex) Flat chuck finger

There are three types of chuck fingers. According to the shape of ring, use chuck finger appropriately to hold the ring and not allow it to become loose or fall.

Flat chuck finger        Round chuck finger (wide width)   Round chuck finger (narrow width)

(3) Chuck finger changing method

Use the chuck finger spanner provided along with the product to release or tighten the chuck finger.

If the thickness of ring is not even, mix different sized chuck fingers to hold it.

(4) Materials holding methods for engraving bracelets

Place three bangle adapters in the rotation clamp, and tighten the chuck finger in front of the bangle adapters. Open the rotation clamp to match it to the bracelet size, and insert the bracelet until it clicks into the chuck finger. Use the chuck handle not to make bracelet come out or spin.

7. Changing tools

Just like the below figure, insert it to 1.5mm wrench on tool fixing bolt. Turn wrench to the left and loosen bolt. Grab tool by hands in advance in order that tool is not fallen on the bottom and pull out tool downward by pushing tool from the top. After changing tool, tighten it by using wrench bolt and fix it

1.5mm wrench

8. Adjusting tension

When marking with a tool, the engraving depth is adjusted with pressure. This is the using same as the engraving depth selection when creating the tool path. If you turn the tension adjustment knob to the right, the pressure increases stronger, and turning it to the left, decreases the pressure.

Program Usage Description

Run the program, then a window will show up as below. The program window is different in accordance with modes. The names of parts for each case are as follows.

◈ Program screen

Work area

Template list

Design

sample list

Menu

Toolbar

Template category

Screen zoom in/out tools

Design sample category

◈ Text input mode

Press  [Text Input] button and click the screen or double click on the selection mode to switch to text input mode.

Cursor (text input mode)

Text property

◈ Object selection mode

After entering text, click [Selection] button with a mouse or right-click the work area. Then, the entered text object is selected and it turns into selection mode. Double click the selection mode to convert it into text input mode.

Object selection mode

Object property

◈ Object rotation mode

If the selected object is clicked once more, the mode will be converted to the object rotation mode.

Object rotation mode

Rotation registration information

1. Toolbar

◈ Alignment tools

[Line/Curve]

|  | Click a certain point and drag it to make a dot line appear. Designate the distance and click with the mouse and then right-click to create a straight line. At this time, press “Ctrl” key on the keyboard and move the mouse to create a horizontal line or a vertical line. |
| --- | --- |
|  | Click on a certain point and designate a distance. Drag while clicking and holding the mouse to make a curve appear. Right-click to create the curve. |

[Circle/Ellipse/Pie]

|  | Click and hold the work area with the mouse and drag it to designate the size of a circle. At this time, press “Shift” key and drag it to create a perfect circle. |
| --- | --- |
|  | Click button on the circle object property window to transform into an arc. The starting/ending degree can be designated and those degrees can be adjusted by dragging the starting/ending degree of the arc with the mouse. At this time, click button to create a reversed image. |
|  | Click button on the circle object property window to transform into a pie (a filled arc). The starting/ending degree can be designated and those degrees can be adjusted by dragging the starting/ending degree of the pie with the mouse. At this time, click button to create a reversed image. |

[Rectangle]

|  | Click and hold the work area with the mouse and drag it to designate the size of a rectangle. At this time, press “Shift” key and drag it to create a square. |
| --- | --- |
|  | Determine the curvature of each edge on the rectangle object property window or drag the rectangular edge with the mouse to transform it into a rectangle with round corners. |
|  | Click button on the rectangle object property window and adjust the curvature of each corner individually. |

[Polygon]

|  | Click and hold the work area with the mouse and drag it to designate the size of a polygon. At this time, specify the number of vertical on the polygon object property window to change it. |
| --- | --- |

[Mirror]

|  | Left/right reverse the selected object. |
| --- | --- |

[Rotation]

|  | If the selected object is clicked once more with the mouse, it will convert to the object rotation mode. Then, rotate the object by dragging it with the mouse or arrow keys on the keyboard. |
| --- | --- |

[Invert color]

|  | Reverse the color of the selected object. |
| --- | --- |

[90˚CW]

|  | Rotate the selected object by 90˚. |
| --- | --- |

[Vertical center in page]

[Horizontal center in page]

[Center in page]

| Alignment |  | Align to the left of base object |
| --- | --- | --- |
|  |  | Align to the center of base object |
|  |  | Align to the right of base object |
|  |  | Align to the top of base object |
|  |  | Align to the center of base object |
|  |  | Align to the bottom of base object |

| Adjust horizontal gap |  | Expand horizontal gap |
| --- | --- | --- |
| Adjust vertical gap |  | Expand vertical gap |
| Same size |  | Make same width |
|  |  | Make same height |
|  |  | Make same size |
| Order |  | Bring to front |
|  |  | Send to back |

◈ Object tools

[Design sample]

Import design sample or save current object as design sample.

What is a design sample?

It means design objects saved in the design sample library. The design sample is easily and quickly imported from the design sample library and freely edited.

[Selection]

It is available either to select the object created in the work area to adjust its size and location or to change the object’s property. Only the selected objects are subject to change their options such as location, size, etc. In the text input mode, convert to selection mode easily by right-clicking the work area.

[Object selected]                                [No object selected]

[Text input]

If  button is pressed and the work area is clicked with the mouse, then characters can be entered. Double clicking in the selection mode is an easier way to convert into the text input mode.

Cursor (Text input mode)

Press  button and drag a character to edit with the mouse for partly selection. Then, adjust its font and size.

[Mouse drag]

[Circular text]

Create text objects along a circle.

[Drawing pen]

Subject can be drawn by dragging the mouse.

The thickness of drawing line can be adjusted in the drawing object property window.

[Outline stroke of object]

The image in a rounding box can be inserted into the target page

[Invert color of overlapped area]

When two or more objects are overlapped, the color in overlapped area can be inverted.

Move the target object over the first object after clicking  button, then overlapped area’s color will be converted.

When converting text, drag the target object over the text after clicking  button.

• [Invert color in overlapped area] function is only applied to straight lines, curved lines, circle, rectangle, polygon, design sample and uploaded SVG file, not texts and images.

• [Invert color in overlapped area] function inverts the color of underlaid object.

If the color is not converted, select [Layout menu → Z Order → Top Most] and place the underlaid object above the other object.

[Image]

Import image (bmp, jpeg, gif, png … etc.).

[Line eraser]

Clear the line of object.

Move the mouse over the line of object, then click the line. The line will be erased (Some objects cannot be erased with the Line eraser tool).

◈ Toolpath

Toolpath?

Toolpath is a pathway where tool goes by for engraving.

Toolpath is usually displayed in red color and engraving tool goes on the pathway as it engraves.

◈ Origin point tools

[Left top origin point]

[Left bottom origin point]

[Center origin point]

◈ Basic tools

[New document]

Open a new file.

[Open]

Open a file in “*.dgn” file format.

Select a file on the “Open” window and click “Open” button.

[Save document]

Save the design drawn on program by “*.dgn”.

Choose the location(folder) in which the file will be saved and enter the file name before clicking “Save” button.

[Print]

Print the design drawn on program.

[Cut]

Cut the selected object.

If Ctrl+X is pressed on the keyboard, the same function will be executed.

Ctrl + V

Ctrl + X

[Copy]

Copy the selected object.

If Ctrl+C is pressed on the keyboard, the same function will be executed.

Ctrl + V

Ctrl + C

[Multiple paste]

If more than one of the same objects is needed, multi-copy the object in several constant gaps by clicking Edit menu → Copy → Multiple Paste….

[Paste]

Paste the cut or copied object.

If Ctrl+V is pressed on the keyboard, the same function will be executed.

[Undo]

Return the work recently done to its previous state.

The number of undo stage can be adjusted with Tool menu → Option → General.

Change font

Undo

[Redo]

Re-execute the undone work.

Redo

Change font

Undo

◈ Screen zoom in/out/move tools

[Zoom in]

The screen is zoomed in by one level.

[Zoom out]

The screen is zoomed out by one level.

[Zoom in/out ratio]

If any screen ratio is selected, the screen in the size of the selected ratio will be seen.

[Magnifier]

Drag a certain spot with the mouse to enlarge it.

[Fit to object]

Enlarge the selected object to make it filled in the screen.

[Fit to page]

Adjust the page size to see the whole work area.

[Panning]

Drag the screen with the mouse to move it.

2. Text object property

If any text object is selected, the property of the text object will appear on the left of the screen.

[Font]

Designate font. Either click the font’s arrow or click “Preview” button to designate font.

Right click to display menu on font preview window. Click “Show Font List”, then you can designate the fonts that will be used in preview.

[Mouse right button click]

[Size]

Adjust the size of the selected text. Either press the size arrow button to select or press  [size] button to adjust the size by one level.

[Bold]

Change text style to bold.

[Italic]

Change the text style to italic.

[Reduce or increase font weight by one level]

Adjust the weight of font.

[Symbol]

Insert symbols.

[Text change]

The entered text of the selected object can be displayed and changed. Edit the text on the text field or press  button to change the text more easily when several text objects are selected.

[Align]

| [Align to left] | Align characters to the left in the text box. |
| --- | --- |
| [Align to center] | Align characters to the horizontal center in the text box. |
| [Align to right] | Align characters to the right in the text box. |
| [Both] | Align characters to the both direction evenly in the text box. |
| [Align to top] | Align characters to the top in the text box. |
| [Align to center] | Align characters to the vertical center in the text box. |
| [Align to bottom] | Align characters to the bottom in the text box. |
| [Horizontal text ] | Arrange characters horizontally. |
| [Vertical text] | Arrange characters vertically. |
|  | Move the selected character horizontally (left, right). |
|  | Move the selected character vertically (up, down). |
|  | Adjust the width of the selected character. |
|  | Rotate the selected character. |

[Position/Size]

|  | Location of the selected object |
| --- | --- |
|  | Size of the selected object |
|  | When “Keep Ratio” is checked, width and height of the selected object is maintained at the same rate. When “Keep Ratio” is un-checked, width and height of the selected object is adjusted separately. |

[Line and Fill Shape]

|  | Line thickness of the selected object |
| --- | --- |
|  | Line type of the selected object |
|  | Line color of the selected object |
|  | Color filled inside of the selected object |
|  | Type filled inside of the selected object |

3. Rotation tool property

[Rotation]

|  | Rotate the selected object by designated degree. Enter the desired angle of rotation, and then press arrow button. The object is rotated by the angle entered. |
| --- | --- |
|  | Rotate the selected object at the same degree as entered. |
|  | Rotate and add a new object at the same degree as entered |

4. How to engrave for each toolpath

Toolpath?

Toolpath is a pathway where tool goes by for engraving.

Toolpath is usually displayed in red color and engraving tool moves on the pathway as it engraves.

[Outline toolpath]

Creates a toolpath that will engrave the selected object’s border.

[Hatching toolpath]

Creates a toolpath that will engrave inside of the selected object.

[Single line toolpath]

Creates a toolpath that will engrave the center line of the selected object.

“Single line toolpath” process may cause the line deviated.

Thus, it is recommended to use thinner lined font to reduce the deviation.

[Boundary-line toolpath]

Creates a toolpath that will engrave boundary line of the selected object.

[Cut-out toolpath]

Creates a toolpath that will cut out the selected object. The toolpath boundary will be wider than the selected object’s boundary depending on the diameter of tool.

[Text impact engraving toolpath]

Creates a toolpath that will engrave inside of the selected object with dots.

[Inline toolpath]

Creates toolpath that will engrave boundaries inside the selected object.

The toolpath is created on inner line that is placed within boundary of the selected object.

Inline

Outline

[V-cut toolpath]

Creates toolpath that will engrave using V-cut tool.

[Photo impact engraving toolpath]

Creates a toolpath that will engrave image with dots.

[Scan line toolpath]

Creates toolpath that will engrave inside area of the target object’s boundary.

“Scan line toolpath” might be similar to “Hatching toolpath”, but it is rather used for logos, patterns or objects with larger area.

-Scan line toolpath-                            -Hatching toolpath-

Better engraving quality is achieved by “Scan line toolpath” when engraving large area, but this may require more time in process.

[Contour line toolpath]

Creates a toolpath that will engrave the selected object with contour lines.

[Photo halftone engraving toolpath]

This converts an image into half-toned and engraves the object in dots or horizontal lines using V-cut tool.

This process is only applied to models with rotating-spindle function like Magic 70, F350P, etc.

[Dot]

[Horizontal Line]

Half-toned engraving?

Half-toned engraving converts light and dark area of a picture or image into lines or dots with different sizes to express fine image.

[Simulate toolpath cut]

The prospected image of the toolpath outcome processed by the selected tool is shown.

The following is an example of the cutting toolpath that uses 0.6mm endmill cutter.

[Show toolpath only]

Only toolpath is shown on the screen with temperately masking all of other objects. If this button is pressed once again, all objects will be seen again.

5. Create Toolpath

Tools in [Tool] option may differ according to types of models.

| Tool |  | Designate a tool to be used for engraving. Each tool setting value can be changed by pressing Edit or Add button. |
| --- | --- | --- |
| Preset |  | Hatching gap, depth, engraving speed and others are pre-set for each engraving option. Therefore, it is not necessary to designate hatching gap, depth, engraving speed and others every time and they will be automatically set with their pre-set values. Each engraving option value can be changed by pressing Edit or Add button. |
| Direction |  | The followings are shown only on hatching tool path. A tool path is created in the horizontal direction. A tool path is created in the vertical direction. A tool path is created in the left diagonal direction. A tool path is created in the right diagonal direction. A tool path is created in the cross hatched direction ※ This toolpath is not recommended due to possible damage to materials with plated surface.. |
| Option | Depth | Engraving: It does not mean the actual engraving depth but refers to “applied pressure” and the higher depth value is, the bigger the applied pressure is. Cutting: Designate an engraving depth actually used for using rotation tools. |
|  | Z Pitch | When cutting the engraving depth little by little, designate the depth. When cutting thick materials with a thin tool, this option is necessary. Z Pitch Actual engraving depth |
|  | Engrave outline | If this option is un-checked, a toolpath for only the inside of the selected object is created. |
|  | Hatching interval | Designate hatching gap. 0.1mm 0.2mm 0.3mm |
|  | Margin | Designate a margin between object outline and toolpath. 0.1mm 0.2mm 0.3mm |
| Speed | XY speed | Designate a speed at which XY axis will move during the engraving. |
|  | Z speed | Designate a speed at which Z axis will move during the engraving. |
|  | Engraving speed is displayed in colors according to tools or engraving options. |  |

| Move Option | Z clearance | This designates the height to be applied for the clearance between the tool and the surface of material when moving to other engraving location. When moving to another engraving location, it is necessary to lift Z axis to prevent the tool from contacting with the material surface before moving. For this, designate the height to be applied at this time. Z Clearance |
| --- | --- | --- |
|  | Z dwell time | This designates no-load rotation time of end-mill and V-cutter after they reach the target depth and before move to X, Y axis point. It is possible to damage the tool as rotation speed reduces while engraving, so set up the Z-dwell time to recover the full rotation speed before engraving. ✽Recommended time: 0.5 ~1 second. |
| Transmit |  | Toolpath output window is displayed |

(1) Tool: Add/Delete/Edit

If “Edit” button is checked, Add/Delete/Edit buttons will appear.

Designate the name of a tool to be added along with its type and other options like tip width, then click “OK” button. The tool will be added to the tool selection list.

Select a tool from the list and click “Delete” button to delete the selected tool from the list.

Select a tool for modifying and click “Edit” button. If “OK” button is clicked after option is modified, the modified value will be saved.

(2) Engraving Option: Add/Delete/Edit

If “Edit” button is checked, Add/Delete/Edit buttons will appear.

Enter a name to the engraving option setting window and designate hatching gap, engraving depth and speed before clicking “OK” button. The option will be added to the engraving option selection list.

Select an option to be deleted and click “Delete” button to delete the option from the list.

Select an option to be modified and click “Edit” button. If “OK” button is clicked after option is modified, the modified setting value will be saved.

6. Toolpath object property

If toolpath object is selected, the toolpath object property window will appear on the left of the screen to allow the user to adjust its depth and speed once again.

| Depth | Engraving: It does not mean the actual engraving depth but refers to “applied pressure” and the higher depth value is, the bigger the applied pressure is. Cutting: Designate an engraving depth actually used for using rotation tools. |
| --- | --- |
| Z pitch | When cutting the engraving depth little by little, designate the depth. When cutting thick materials with a thin tool, this option is necessary. Z Pitch Actual engraving depth |
| XY speed | Designate a speed at which XY axis will move during the engraving. |
| Z speed | Designate a speed at which Z axis will move during the engraving. |

| Z clearance | This designates the height to be applied for the clearance between the tool and the surface of material when moving to other engraving location. Z Clearance When moving to another engraving location, it is necessary to lift Z axis to prevent the tool from contacting with the material surface before moving. For this, designate the height to be applied at this time. |
| --- | --- |
| Z dwell time | This designates no-load rotation time of end-mill and V-cutter after they reach the target depth and before move to X, Y axis point. It is possible to damage the tool as rotation speed reduces while engraving, so set up the Z-dwell time to recover the full rotation speed before engraving. ✽Recommended time: 0.5 ~1 second. |
| Engrave twice | Engrave the design twice. |

7. NC data output

Select toolpath and click engraver button on the top-right of screen to transfer the selected toolpath to the engraver.

This button is to exclude other models connected to the computer.

Select the communication port to be excluded and click OK.

When engraver detection problem is met, click this button to retry the detection.

When the engraver is identified, the model name will appear on the button.

“NC output” window appears when this engraver button is clicked.

On the NC output window, select a measuring method and other options, then click “Start Engraving” button to start transferring the toolpath to the selected engraver.

| Toolpath order | When two or more toolpaths are selected for engraving, they will be placed in order. The order can be adjusted by [Up], [Down] button on the right. |
| --- | --- |
| Surface height | Measure material height. |
| Multi-point-measurement | Measure material height by dividing the whole material into several parts(up to 5 times) |
| Outside | Engrave on the ring’s outer diameter. |
| Inside | Engrave on the ring’s inner diameter. |
|  | Place the material to the flat chuck finger. |
|  | Place the material to the round chuck finger. |
| Multiple engraving | Engraves materials repeatedly in constant vertical/horizontal gap. ✽Usually optional vise is used for this automatic engraving. |
| Simulate engraving | This displays engraving simulation on the material by using laser pointer to assure its correct pathway. |
| Adjust engraving position | Designate whether to re-adjust the engraving location. If this option is selected, the laser pointer will be turned on to allow the user to adjust the engraving location before engraving. If this option is not used, engraving will initiate immediately. |
| Move to vise automatically | Automatically moves to the flat or rotating vise in accordance with selected template. |

Engraving Process of Ring inner/outer Diameter

Tool: Tip

Clamp: Rotation clamp

◈ Preparation

1. Measure the width and diameter of a ring to be engraved.

Width

Diameter

2. Insert the ring into the chuck finger as below, and tighten the rotation clamp to set it.

[Outer diameter engraving]        [Inner diameter engraving]

Since the engraving direction is as shown in the picture below, mind the direction of the material and set it.

AB

◈ Program Process

1. Double click “Ring” template in the object property window.

2. When “Insert Rotary Object Template” window appears, enter the measured width and diameter of the ring and click “OK.”

3. The template appears at the center of the screen.

4. Select [Text Input] on the toolbar, and click the work area using mouse and enter text to be engraved.

5. After entering text, press [Selection] on the toolbar to enter the object selection mode. Select the text, and edit the size and font of the object. To place the object at the center of the work area, click [Center in Page] button.

Handle to adjust character gap

Handle adjusting gaps between characters?

A function adjusting gaps between characters by dragging characters to target positions.

6. Once design is completed, click [Hatching Toolpath] button.

7. Once “Hatching Toolpath” window appears as shown in the picture below, select tool, toolpath gap and depth, and click “Create Toolpath” button.

8. The “Toolpath” object is created as below and usually marked with red line on the screen. Scroll the mouse wheel to enlarge the toolpath object, and if it is not the desired toolpath, delete the toolpath and create it again by adjusting the toolpath option.

[Toolpath creation]

9. After selecting one or more created toolpaths, click  button.

10. Once “NC Output” window appears, specify how to measure the engraving materials. And then, click “Start Engraving” button.

| Surface height | Measure material height. |
| --- | --- |
| Multi point measurement | Measure material height by dividing the whole material into several parts(up to 5 times) |
| Outside | Engrave on the ring’s outer diameter. |
| Inside | Engrave on the ring’s inner diameter. |
|  | Set the material to the flat chuck finger. |
|  | Set the material to the round chuck finger. |

11. Once the tool is moved to the center of the material and located, the engraving area can be verified and adjusted by using the direction key button. At this time, check the laser pointer while moving it for easier measurement and adjustment.

|  | Whenever the direction key is pressed, move the laser pointer to the left, top, right and bottom of the engraving area to show the area to be engraved. Check whether the area to be engraved is outside of the materials surface. If so, adjust the location by using the direction key. |
| --- | --- |
|  | Work area The laser pointer moves the area to be engraved to the left, top, right or bottom according to current location. If the current location of the laser pointer is left, then, press key to move the area to the left. |

12. After the location of engraving is decided, press and hold  button until it beeps (more than 3 seconds) to start the engraving and close the engraver cover.

Engraving Curved Materials

When a material to be engraved is curved, engraving process may scratch the surface of the material. Entering the thickness of curved surface to be engraved to automatically set up the Z moving height can prevent from scratching the surface.

1. Create a toolpath and Click  button.

2. Once “NC Output” window appears, click “Uneven” to show various kinds of the curved material. Select a shape that is the most similar to that of the material to be used and enter the thickness (thickness of the area to which the engraving is applied) of the material’s round part.

Enter the thickness of the part to be engraved.

Layout of Text in a Circle

Text can be laid out in a circle by using circular text object.

[Circular Text]

Click  button to display “Circular Text” window.  Designate direction and angle, enter size and text, then click “OK” button.

[Circular Text (Top/Bottom)]

Click  button to display “Circular Text (Top/Bottom)” window. Enter the size of outer circle, inner circle and text and click “OK” button.

Adjusting layout angle

Drag the first character [R] or last character [y] in the text to adjust the angle of the layout.

Drag with mouse to adjust the layout angle.

Drag other characters with mouse to adjust the distance to the center of the circle.

Drag with mouse to adjust distance from the center of the circle.

Layout of Text on a Curve

1. Click the [Line/Curve] button.

2. Click on a certain point with the mouse and designate a distance. Drag the mouse to make a curve appear. Press the right button on the mouse to create the curve.

[Mouse drag]

3. Select “No Line” on the “Line and Fill Shape” window on the object property window.

4. Click [Text Input] button, and move the mouse cursor to the curve line. The cursor shape will change as below.

5. In state of the changed cursor shape, click to enter the text input mode.

Font Preview

Apply the fonts installed on the computer to the text objects and preview the shape to select the most suitable font.

Select text object, click “Preview” button in the object property window or right-click the mouse and select “Font Preview” menu.  Then, “Font Preview” window will appear.

Drag the font list in the font preview window, or click a font while pressing “Ctrl” or “Shift” key, then the preview of the selected font appears automatically. Double click the preview image of the font to select it.

[Mouse right button click]

Text Change

The text contents in text objects can be easily changed by the “Text Change” feature.

Import a saved design sample, file, or select text objects designed on the screen. Click [Text change] button or right- click the mouse, and select “Text Change” on the menu.

Change contents in the “Text Change” window, and click “OK” button. The changed texts will be shown on the screen.

Template Creation

Template is an image corresponding to the materials to be engraved by 1:1 ratio. Load a template that is the most similar to the material to be engraved, and enter the text to be engraved. This way allows you to create the location of engraving and the size of text to be engraved more easily and conveniently. Circle, heart, square and diamond-shaped templates are initially provided by the program and users can freely add more templates.

The shape of a template can be designed by graphic program (Illustrator, Photoshop, CorelDraw and etc.) and loaded by SVG file format. Also scanned image file such as jpeg, bmp, png, etc. can be applied as template.

[Creating template #1]

1. Select Tool menu → Template → Create Template from Image.

2. "Create Template from Image” window appears.

3. Select image files (jpeg, bmp, png and etc.) from the “Open” window, and click “Open” button.

4. Input size of the template.

Check “Save” to save the loaded image as a template for later use and enter a name for the template.

Click “OK” button to load the image as a template.  If “Save” is checked, and the template name is given, the loaded image will be saved as a template in the template library simultaneously.

✽ Right-click in the template category window for more template related features. The template can be deleted by “Delete” on the menu that appears by right-click in template list window.

[Mouse right button click]

[Creating template #2]

1. Press [Image] button to import an image.

2. Select Tool menu → Template → Save Selected Object as Template.

Type a name of new template on “Input” window.

3. The saved template is added in the template library.

The template saved in the “Save Selected Object as Template” menu is saved as designated size to the screen. When loading the template, check “Maintain Original Size”, then the template will be loaded with the original size designated when saving the template.

Saving as Design Sample

Edited contents can be saved as design sample for easier access to the design.

[Saving as design sample #1]

1. Edit contents and select the objects.

2. Click File menu → Save as Design Sample and select a folder or create new folder.

Enter the name and click “Save” button to save it.

[Saving as design sample #2]

1. Select objects, and click [Design Sample] button.

2. Select a folder to save the objects from design sample library window.

3. Click [Save selected image as sample] or right-click the design sample image, and click “Save Selected as Design Sample” menu.

Enter the name to be saved, and click “OK” button to save it in the design sample library.

[Mouse right button click]

Auto Change of Serial Number

Use this function when changing numbers by certain unit.

Edit the font and size of text object with number, and press “Alt+N” key on a keyboard, or select Text menu → Serial Number → Advance Serial Number to change the number automatically.

Select Text menu → Serial Number → Serial Number Option and enter a unit from the “Serial Number Auto Advancing Option” to change the serial number by the designated unit automatically.

If the serial number is not regular, then select Text menu → Serial Number → Load Serial Number.

Enter serial number in the “Lines to export” field or click “Load File” button to load it from a file (.txt or .csv file format).

Once serial numbers are loaded, click “Export to object” button. The text on the selected text object is automatically changed, and the serial number is deleted from the list. The deleted numbers will be shown in the “Exported Lines”.

Serial numbers can be loaded from comma separated values file (.csv) or text file (.txt).

Each row of the file is recognized as one serial number, and the contents of each row separated by comma are shown in the next line.

Loading and Editing Image

1. Loading an image

This program provides a function to load various types of image files. Supported image file types are as follows.

| BMP | Windows or OS/2’s bitmap graphic file |
| --- | --- |
| GIF | CompuServe graphic file |
| JPG/JPEG | JPEG bitmap graphic file |
| PNG | Portable Network Graphics bitmap graphic file |
| PCX | Z Soft PC paintbrush bitmap file |
| TIF/TIFF | Tagged Image File Format bitmap file |
| WMF | Windows meta file |

Image file can be loaded by following method.

1. Select [Image] on the toolbar or select Insert menu → Image.

2. When “Open” window appears as the figure below, select the image to be loaded, and click “Open” button.

3. Drag the work area with the mouse to load the image by desired size.

2. Converting and editing image

Only single color image can be engraved. Since multiple color images cannot be engraved by engraver, it should be converted to single color (1 bit) image.

[Converting to single color image]

1. Select the image object you want to convert, and click “Convert” button in the image object window and select “Monochrome (1bit).”

Or select the Transform menu → Bitmap → Mode → Monochrome (1bit).

2. “Color Image to Black & White” window appears as below. Adjust the slider to change it to the most suitable single color, and click “OK” button.

[Editing image]

Loaded image can be edited by “Paint” program in Windows.

Select the image object to be edited, right-click it to select “Edit” or select Transform menu → Bitmap → Edit.

Click [OK] to run Windows Paint program.

Then, Windows Paint program will show the image.

After editing the image, press “Save” from the File menu of Paint, and close the program to apply the edition.

Import a File in SVG Type

Loading vector graphic files such as SVG file is also supported.

The SVG file is created by various graphic programs such as Illustrator, CorelDraw and etc.

1. Select File menu → Save As from Illustrator program to save a file in SVG type.

2. SVG option window appears. Press “OK” button to complete the save.

3. Click  button to load the SVG file.

4. Or select File menu → Import to import a file.

Origin point calibration

※ The machine is initially set up at factory. Please contact reseller before changing the machine settings.

※ Calibrate the machine by using tip(Carbide, Diamond). The machine cannot be calibrated by using endmill, cutter, etc.

※ Calibrate the machine only when Engraver pressure lock is not fixed. The nose, tool, and material can be damaged if you calibrate the machine when the head is fixed.

1. Tool calibration

1. Turn on the engraver.

2. Turn the chuck handle to narrow the chuck finger’s gap as much as possible.

3. Press and hold  button for more than 2 seconds to move the tool to the clamp.

4. Turn the chuck handle as shown in the picture below to vertically align the tip of the tool and chuck finger.

5. Turn the right part of the chuck handle until the tip touches the surface of the chuck finger.

6. Press and hold  button for more than 2 seconds to save the settings.

2. Laser pointer calibration

Laser Radiation

Avoid eye or skin exposure to direct or scattered radiation. Class IIIA laser product.

1. Fix a ring on flat chuck finger to the rotation clamp of the engraver.

2. Press and hold  button for more than 2 seconds.

3. The tool draws cross (＋) lines on the ring and the laser pointer is turned on.

4. Move the laser pointer to the center of the cross by using thedirection key. Then press and hold button.

5. Laser pointer moves closer to the material to accurately calibrate the laser pointer’s origin.

Move the laser pointer to the center of the cross by using the direction key on engraver, and press and hold the  button until it beeps (more than 3 seconds) to confirm that the setting is saved.

3. Rotation clamp starting point (Z axis)

1 Set the sharp tool into the rotation chuck of the engraver as shown in the picture below.

2. Press  button on the engraver to move the tool to the center of rotation clamp.

3. Use Z axis  buttons to align the end of the ring inner diameter marking tool and tip of the sharp tool on the same horizontal line. If the distance between the tool and the sharp tool is too far, then bring them closer by using  button.

4. Press and hold  button for more than 2 seconds to save the settings.

4. Chuck finger starting point

1. Set the chuck finger to the rotation clamp of the engraver.

Flat chuck finger              Round chuck finger

2. Press  button to turn on the laser pointer at the starting point of the chuck finger.

3. Use the direction key of the engraver to match the laser pointer to the base point of each chuck finger as below.

4. Press and hold  button for more than 2 seconds to save the settings.

R&D by REDT

http://www.redt-magic-engraver.com

3F, (1314 Gwanpyeong-dong) 13-10, Techno 2-ro, Yuseong-gu,

Daejeon, Republic of Korea, 305-709

TEL: +82-70-7011-0905 FAX: +82-42-673-0905